PART 1: The Challenge:

Background: Safely Riding a Car.

You are tasked to design a combinational logic circuit that operates an alarm in a car whenever the driver and/or passenger seats are occupied, and the seatbelts are not fastened when the car is started. The active-HIGH signals DRIV and PASS indicate the presence of the driver and passenger, respectively, and are taken from pressure-actuated switches in the seats. The signal IGN is active-HIGH when the ignition switch is on. The signal 𝐵𝐸𝐿𝑇𝐷̅̅̅̅̅̅̅̅̅̅ is active-LOW and indicates that the driver’s seatbelt is unfastened. 𝐵𝐸𝐿𝑇𝑃̅̅̅̅̅̅̅̅̅ is the corresponding signal for the passenger seatbelt.

The alarm will be activated (LOW) whenever the car is started and either of the front seats is occupied and its seatbelt is not fastened. Your goal is to design the logic circuit needed to fulfill the above conditions. You must use the problem-solving process (PSP) taught during the lectures alongside the Boolean design process (BDP). Below is a step-by-step guide to assist you in this endeavour. Your tutors will also assist you in this.

Step 1: Understand and Define the Problem (Analyse)

Design a combinational logic circuit for a seatbelt alarm in a car. The alarm will be bases off the driver and passenger seat belts and whether they are fastened. The active HIGH signals DRIV and PASS will be used to indicate whether there is a driver or passenger. These will be gathered from pressure-actuated switches in the seats. Signal IGN is active-HIGH when the ignition is on. The BELTD is active is active-LOW and indicates that the driver’s seatbelt is unfastened. 𝐵𝐸𝐿𝑇𝑃̅̅̅̅̅̅̅̅̅ is the corresponding signal for the passenger seatbelt.

Step 2: Organise and Describe the Data

| Symbol | Description | Type | Example |
| --- | --- | --- | --- |
| DRIV | Driver is in seat | Input | High(1=present) |
| PASS | Passenger | Input | High(1=present) |
| IGN | Ignition is on | Input | High(1=on) |
| BELTD | If driver is unfastened | Input | Low(0=unfastened) |
| BELTP | If passenger is unfastened | Input | Low(0=unfastened) |
| Alarm | If above conditions not satisfied | Output | Low(0=alarm on) |

Step 3: Plan the Solution (Design the Algorithm)

3.1 – Algorithm

- Monitor Ignition switch – IGN must be HIGH

- Check if driver is seated and drivers’ belt is not fastened

- Check if passenger is seated and belt is not fasted

3.2 – Truth tables

| Inputs |  |  |  |  | Outputs |
| --- | --- | --- | --- | --- | --- |
| DRIV | PASS | BELTD’ | BELTP’ | IGN | ALARM |
| 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

3.3 – Boolean Expressions (SOP)

DRIV . PASS . BELTD .BELTP .IGN

DRIV . PASS . BELTD .BELTP .IGN

DRIV . PASS . BELTD .BELTP .IGN

DRIV . PASS . BELTD .BELTP .IGN

DRIV . PASS . BELTD .BELTP .IGN

ALARM= DRIV . PASS . BELTD .BELTP .IGN+ DRIV . PASS . BELTD .BELTP .IGN+ DRIV . PASS . BELTD .BELTP .IGN+ DRIV . PASS . BELTD .BELTP .IGN+ DRIV . PASS . BELTD .BELTP .IGN

3.4 - Pseudocode

IF (DRIV .(PASS) ̅  .(BELTD) ̅  .(BELTP) ̅  .(IGN) ̅) OR (DRIV .(PASS) ̅  .(BELTD) ̅  .BELTP .(IGN)) OR (DRIV .PASS .(BELTD) ̅  .(BELTP) ̅  .(IGN) ) OR (DRIV .PASS .(BELTD) ̅  .BELTP .(IGN) ) OR (DRIV .PASS .BELTD .(BELTP) ̅  .(IGN)) ̅

Alarm is LOW # ON

ELSE

Alarm is HIGH # OFF

Step 4: Test and Refine the Solution

| DRIV | PASS | BELTD’ | BELTP’ | IGN | ALARM |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |